Without an AI-agent, the common way to start running DualSPHysics is as follows:

- Pre-Processing – XML file generation:

First an xml file is created that stores the initial geometry and physical and numerical settings of the simulation. We can use /examples/mphase_nnewtonian/DebrisFlow2D/CaseDebrisFlow2D_Def.xml as an example. There are two ways to generate the xml file: 1) one may download FreeCAD and load a specific DualSPHysics module called DesignSPHysics (https://github.com/DualSPHysics/DesignSPHysics). Once DesignSPHysics is open, one may define geometry and other parameters using that user interface. 2) manually create the xml file by following all the example xml files in examples/ and/or guides. The guides include files in /doc/guides like DualSPHysics_v5.0_GUIDE.txt, XML_GUIDE_MDBC.pdf, XML_GUIDE_MPHASE_NNEWTONIAN.pdf, XML_GUIDE_v5.0.pdf. The rest of the files provide other functions that may not be needed for now; the guides also include all the files in /doc/xml_format. The role of the Pre-processing agent is to: bypass the xml file generation manually or from DesignSPHysics, generating xml files from natural language input; or if an xml file has been created (no matter manually or from DesignSPHysics or from the agent), the agent can communicate with humans to allow checking and receiving consent for running the next step.

The pre-processing agent should be able to perform in the following example scenarios:

- Human input: “hey, I want to create a case to simulate water running after a dam is broken. The geometry is xxx, the water properties is xxx, I want to run until xxx simulation stoppage criterion”. The pre-processing agent should create an xml file based on what it learnt, and call Paraview to ask human to check and receive command for further revision or run the next step.

- Human input: “hey, I have an existing xml file at xxx location. What I want to simulate is xxx. Can you modify the xml file if necessary and let me check?”. The pre-processing agent should revise the xml file as needed, and call Paraview to ask human to check and receive command for further revision or run the next step.

- Human input: “hey, after your revision, I feel like this is too complicated a scenario that you could not fully understand. Therefore, I manually corrected your generated xml file, can you call Paraview to let me check?”. The pre-processing agent should call Paraview, after receiving consent, continue to the next step.

Documents help create xml file that the AI-agent should learn from:

- As previously stated, the /guides/, /help/ and /xml_format/ folders in /doc/.

- All the xml examples in /examples/ folder. Learning should be emphasized on the /mphase_nnewtonian/ folder since we are dealing with Non-Newtonian flow problems, these examples are more relevant.

- The online DualSPHysics wiki at https://github.com/DualSPHysics/DualSPHysics/wiki. Specifically, in the section of “SPH formulation” (https://github.com/DualSPHysics/DualSPHysics/wiki/3.-SPH-formulation), it explains some key concepts that are relevant to how to assign variables in xml file, for example, the concept of artificial viscosity in section 3.2.1, shifting algorithm in 3.6, time stepping in 3.7, boundary conditions in 3.8, Non-Newtonian formulations in 3.17.

Below I will use /examples/mphase_nnewtonian/DebrisFlow2D case as an example to explain the xml file format but if the agent learnt well from the three types of documents listed above, it should be able to create an xml file itself.

Inside CaseDebrsiFlow2D_Def.xml, the first block <constantsdef> creates essential constant parameters. Meaning of many of the parameters are straightforward after reading the “comment”. <speedsystem>, <coefsound>, <sopeedsound> are relevant to the setting of speed of sound, it should be set wisely based on the AI-agent’s understanding of the physics. The speed of sound should be set generally close to reality but in many occasions it is not easy to estimate a reasonable value if the material does not behave elastically. Numercical considerations should also be included because too high a speed of sound may reduce time step too much and cause other problems; too low a speed of sound is also not desired. <coefh> is the smoothing length, one of the most important parameter in SPH. If <coefh> is too low the simulation is not accurate, if it is too high it may causes some instability because a certain kernel cannot handle too many particles inside.

The next block is <geometry>, the agent can refer to the three types of document to generate the initial geometry of the boundary and fluid phases. <dp> defines initial particle spacing, which serves as a resolution purposes. Too have a value will make the simulation too slow and too small will loss resolution. <dp> and<coefh> jointly controls the kind of “mesh dependence convergence” in SPH although SPH is meshless. In FEM, people check different meshing to confirm the numerical convergence of the simulation, in SPH these two parameter can serve the similar role. “mk” for each phase is also defined that is very important, it is the id of the each phase. Usually the fluid phase is set as mk=”0”; if multiple fluid phases exist, then the “mk” can be sequentially increased. Boundary phases are defined in “setmkbound” and usually we set the values after fluid phases. In cases that the geometry is too complicated that is not easy to be defined in xml directly, one may input a stl file. For example, in examples/mphase_nnewtonian/06_Impellers3D, an impellers.stl is created beforehand, and it is inserted into CaseImpellersNN_Def.xml in the line <drawfilestl file="impellers.stl">. Also in the <geometry> block, one can define <shapeout file => which will direct what kind of preprocessing output files will be generated for checking in Paraview.

In <nnphases>, the parameters are Non-Newtonian physical parameters. If the task is to know the rheology of an existing material, then simulations should be run by changing those parameters and find a set of good parameters such that the material rheology is consistent with the existing data.

Some other blocks like <initial>,<floating>,  <motion> are not included in this CaseDebrsiFlow2D_Def.xml but they can be learnt in the three types of learning resources listed above.

The block <parameters> lists other simulation related non-physical parameters. Their meaning are straightforward following the “comment”. For non-Newtonian flow simulation, <RheologyTreatment> should be chosen as “value = 2”. <Visco> here is the artificial viscosity for mitigating numerical instability, don’t confuse this with <visco> defined in <nnphases> (that is the physical viscosity of the rheological flow). <TimeMax> and <TimeOut> controls the time of data output and the simulation stoppage time.

- Pre-Processing – run initialization using XML information:

When an xml file is generated, usually people we call the .sh or .bat file to run. For example, in DebrisFlow2D example, there are four bash files CaseDebrisFlow2D_linux64_GPU.sh or .bat, or GPU code. The AI-agent can learn from all the .sh or .bat file in all the examples/ folder. The folder /doc/help also contains relevant information on how the command should be formulated in the bash file.

Using DebrisFlow2D as an example,  in CaseDebrisFlow2D_linux64_GPU.sh, paths for the binary files and the postprocessing tools are provided. The AI-agent should learn to include the necessary files and ignore unnecessary files for following running. For example, if isosurface tool (what the isosurface is doing can be found in /doc/help/IsoSurface_Help.out) is not needed, then the AI-agent generated .sh file does not necessarily to include the line “export isosurface="${dirbin}/IsoSurface_linux64"”. After the pahts are defined, the next a few lines check if the data folder is exist or not and choose to delete or post-process the existing data or stop the running. Then, when {gencase} is called, some vtk and other files for the initial state of the simulation will be generated, and this is a good time that the A-agent calls Paraview for human to review and give further directions. Again, detailed GenCase related command can be found in /doc/help/ folder, which is the file GenCase_Help.out. So, the AI-agent should create such an sh. File and call it “bash xxx.sh” to run the pre-processing. The current .sh files like CaseDebrisFlow2D_linux64_GPU.sh do pre-processing, running, and post-processing in one file. I would suggest the AI-agent to create three files separately. And just call the pre-Processing.sh for the pre-processing steps.

- Running DualSPHysics

After the initial states has been confirmed through human-agent interaction, then the AI-agent can create a main_run.sh file which simply just do the role similar to

${dualsphysicsgpu} -gpu ${dirout}/${name} ${dirout} -dirdataout data -svres

if [ $? -ne 0 ] ; then fail; fi

In CaseDebrisFlow2D_linux64_GPU.sh. Depending on if the GPU or the CPU code is called and other functions, the commands can be different. Refer to /doc/help/DualSPHysics5.0_Help.out, DualSPHysics5.0_NNewtonian_Help.out.

If there is any issue the AI-agent found during running, the agent should learn how to resolve it, or report any warning in the end or if it cannot resolve it itself, call for human interaction.

- Post-Processing

After running the main code, a /data/ folder should be generated that contains all the raw data at each designated output time step. Then, the AI-agent will create a post-processing.sh file and call it to post-process the code. You may also combine the main_run and post-processing if you wish. Take CaseDebrisFlow2D_linux64_GPU.sh as an example, {partvtk}, {partvtkout} and {isosurface} are the postprocessing tools that can be called to convert raw data into human readable .csv or .vtk or other files. The command how to call the tools can be found in /doc/help/. The AI-agent should be able to figure out what post-processing it want to do, or when unclear, communicate with human. For example, if I want to know the run-off distance of the debris flow, the AI-agent may post-process the data, extract the furthest point from the simulation as a function of output time, then plot and save the distance-time data in a proper format.

- Example 1 – solving inverse problem to determine debris flow non-Newtonian parameters.

A ground truth will be given that is a time series profile of a debris flow. The ground truth data could be in a series of image format, or movie, with proper scales, or a series data point files stored in .txt or other formats. Human will input by giving the initial geometry of the domain and the undeformed soil mass geometry. Human will also give some rough estimation of the material state, for example, if given that this is a fine grain soil mass with a large amount of mixed water, then the AI-agent can estimate that the debris flow has a low density and viscosity. The AI-agent will then starting from a guess of the non-Newtonian parameters, run the simulation, post-process the data and compare the time series profile of the simulated soil with ground truth. The AI-agent should give a score of how close the simulation is to the ground truth, and if the deviation is large, it should automatically propose how to change the non-Newtonian parameters and perform another set of simulation. Finally, after many runs the AI-agent should be able to conclude what are the best set of parameters for this debris flow and what are the variation range of the parameters. For example, the AI-agent may conclude that for the viscosity parameter, the debris has a viscosity of 0.1 Pa*s with ± 10% variation gives a good fit. This is what we called inverse problem – solving the debris flow constitutive model parameters given a rheological behavior data.